Faculty of Engineering and Technology

Electrical and Computer Engineering Department

Computer Communication Lab

ENEE 4113

Experiment No. 2 pre-lab

SSB DSB Amplitude Modulation

Prepared by:

Abdel Rahman Shahen 1211753

Partners:

Mahmoud Awad

Instructor: Dr. Ashraf Rimawi

Section: 2

Date: 3/6/2024

# Title: Theoretical Analysis of Amplitude Modulation and Demodulation: DSB-SC and SSB Methods

## Introduction

Amplitude modulation (AM) stands as a cornerstone in the realm of communication systems, facilitating the transmission of audio and video signals across various mediums. This paper introduces the nuanced techniques of Double Sideband Suppressed Carrier (DSB-SC) and Single Sideband (SSB) modulation, elucidating their pivotal roles and diverse applications in modern communications.

## Objective

The study aims to dissect and elucidate the mathematical intricacies and theoretical constructs underpinning AM, DSB-SC, and SSB modulation and demodulation processes, enhancing the comprehension of their operational principles and efficiency.

## Theoretical Background

### Amplitude Modulation (AM)

Amplitude modulation is characterized by the variation of a carrier signal's amplitude in accordance with the message signal, denoted mathematically as m(t)cos(Wc t), where m(t) is the message signal and Wc is the angular carrier frequency.

### Double-Sideband Suppressed Carrier (DSB-SC)

#### DSB-SC modulation, a variant of AM, diverges from the classical approach by eliminating the carrier frequency, conserving power and bandwidth. The modulated signal is mathematically depicted as s(t)=m(t) cos (Wc)

#### Spectrum Analysis:

The DSB-SC spectrum conspicuously lacks the central carrier component, featuring only the sidebands bearing the message information.

#### Mathematical Representation:

The output of a DSB-SC modulated signal can be represented as s(t) = m(t) cos (Wct)

### Single-Sideband (SSB)

SSB modulation refines the DSB-SC technique by further omitting one of the sidebands, resulting in either an Upper Sideband (USB) or a Lower Sideband (LSB) transmission. The USB, for instance, is represented as S(t)=Re {m^(t) e^ (j Wc t)}, where m^(t) is the Hilbert transform signal.

#### Spectrum Analysis:

SSB showcases superior spectral efficiency by occupying only half the bandwidth of DSB-SC and traditional AM, thereby enabling more economical use of the frequency spectrum.

#### Mathematical Representation:

The SSB signal can be derived by filtering one sideband from a DSB-SC signal. Two forms exist, USB (Upper Sideband) and LSB (Lower Sideband).

Equation for USB: S(t) = Re{m^(t) e^ (j Wc t)}, where m^(t) is the Hilbert transform

# Title: practical Analysis of Amplitude Modulation and Demodulation:

## SSBSC:

## SSBSCL: